Changes in this version:

- New SOP template

- New labelling

- Etc…

## INTRODUCTION/PURPOSE

This is a procedure including a detailed risk assessment for making 6 % Perchloric acid. 6 % Perchloric acid is used for EtOH concentration determination.

## NECESSARY SAFETY EQUIPMENT

|  | Nitrile |  | Fume hood |
| --- | --- | --- | --- |

## CHEMICAL AND BIOLOGICAL HAZARDS

### Chemicals

| Chemical information | Health -, Precautions - and Emergency planning |
| --- | --- |
| Perchloric acid 70% 311421 from Sigma CAS no: 7601-90-3 | H271: May cause fire or explosion; strong oxidiser H290: May be corrosive to metals H302: Harmful if swallowed H314: Causes severe skin burns and eye damage H373: May cause damage to organs through prolonged or repeated exposure P210: Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. P280: Wear protective gloves/ protective clothing/ eye protection/ face protection. P303 + P361 + P353: IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/shower. P304 + P340 + P310: IF INHALED: Remove person to fresh air and keep comfortable for breathing. Immediately call a POISON CENTER or doctor/ physician. P305 + P351 + P338: IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P371 + P380 + P375: In case of major fire and large quantities: Evacuate area. Fight fire remotely due to the risk of explosion. |

## Special cautions necessary due to reproductive toxicity:

Generally, it is not recommended to work with a chemical that has carcinogenic or reproductive effects if you are planning to be or are pregnant. If a chemical is proven to pass into breast milk it is not recommended to perform procedure if you are breast feeding.

Planning pregnancy (men and women): None

Pregnant: None

Breast feeding: None.

## PROCEDURES and risk assessment

### Procedure

Necessary equipment:

Pipettes

Pipet tips, assorted sizes

50 ml glass bottle

MQ water

Magnetic stirrer

We always wear lab coat when working in the lab.

- Add approx. 40 ml with dH2O to a 50 ml glass bottle.

- Transfer 4.3 ml 70 % perchloric acid to a 50 ml glass bottle.

- Adjust the volume to 50 ml with dH2O (6 % perchloric acid).

| Part of procedure | Unwanted scenarios | Necessary precautions | S*K (Probability*Consequence) |
| --- | --- | --- | --- |
| 1-3 | Spillage of hazardous chemical | Wear gloves, goggles and lab coat. Work in fume hood Add most of water before adding the acid Locate nearest fire extinguisher | 2*3 |

- Mix on magnetic stirrer.

- Label the bottle with 6 % Perchloric acid and hazard symbol according to the risk conclusion , date and your name and store in the fridge.

| 4-5 | Spillage of hazardous chemical | Wear gloves, goggles and lab coat. Work in fume hood | 2*2 |
| --- | --- | --- | --- |

### Labelling of new solution:

Please find relevant classification information from existing msds or confer with the ECHA guide for the correct classification and labelling of the solution (this might be quite time consuming).

| Perchloric acid 6% (Information from Sigma 34288) | H315: Causes skin irritation. H319: Causes serious eye irritation P280: Wear protective gloves/ protective clothing/ eye protection/ face protection. P305 + P351 + P338: IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. P337 + P313 I If eye irritation persists: Get medical advice/ attention. |
| --- | --- |

## WASTE DISPOSAL

When making this solution some waste is generated. It is important to access the potential risk this on the environment and how this waste should be handled.

| Waste |  | Volume | Disposal method | Environmental risk |
| --- | --- | --- | --- | --- |
| ##### 1 | 70 % perchloric acid Disposal of chemical; too much volume taken out from bottle, the stock solution of 70% perchloric acid is out of date etc. | 0.1 – 100 ml | See chemical waste procedyre (Farlig avfall) | ##### Chemicals for disposal are collected twice a year. According to procedure and handled with trained staff and collected by professionals. |
| ##### 2 | Contaminated disposables or small chemical spillage with 70% perchloric acid; | 0.1 – 1 ml | Risk waste | ##### The risk waste is handled by professionals |
| 3 | 6 % perchloric acid Rest of solution that is too old. Rest for using solution to measure EtOH concentration | up to 50 ml | Risk waste This is rescribed in separate SOP | - |

## REFERENCES